APPROVED BY:

Director

Name LLC

______________ I.P. Surname

Seal

dated 13 October 2026

PROCEDURE

FOR ACTIONS BY THE PERSONNEL OF NAME LLC

IN THE EVENT OF A FIRE

| № # | Action | Steps (what to do) | Responsible person |
| --- | --- | --- | --- |
| 1 | Reporting a fire upon discovery | 1. Immediately report the fire to the State Fire and Rescue Service by calling 101. State the address of the premises, the number of floors in the building, the location where the fire started, the current situation, whether people are present, and your surname. Example report: ‘A fire has occurred at 15 Myru Street. The administrative building is on fire. The fire started in the security room on the ground floor. The fire has now spread throughout the room. The ground floor is heavily filled with smoke. There are people in the building. Reported by: maintenance technician I.P. Surname.’ 2. If the building has a fire alarm system, activate it. If there is no alarm system, announce loudly: ‘Alarm, fire. Everyone must leave the building immediately.’ 3. Notify the management or responsible persons of Name LLC of the fire. 4. Confirm that the Fire and Rescue Service has been called (repeat the report if necessary) and inform the head of the company of the fire. 5. Check whether the occupant warning system, fire-extinguishing systems, smoke-control systems, and other fire protection systems have activated. 6. If necessary, call other emergency services: the gas emergency service (104), emergency medical services (103), the National Police (102), etc. | The first person to notice signs of fire Responsible official |
| 2. | On-site firefighting | 1. Begin extinguishing (containing) the fire using the available firefighting equipment. 2. If necessary, disconnect the electricity supply (except to fire protection systems), stop process equipment, shut off gas and water utilities, stop ventilation systems in the emergency room and adjacent rooms (except smoke-control devices), and take other measures to prevent the fire from spreading and the building from filling with smoke. 3. Organize firefighting using the company’s available personnel and resources. Note: begin extinguishing the fire only if there is no direct threat to human life or health. Otherwise, evacuate personnel only and wait for the emergency services to arrive. | The first person to notice signs of fire. Responsible official |
| 3. | Evacuation of people and material assets | 1. Stop work in the building where the fire has occurred (except work related to firefighting measures). 2. Organize the evacuation of people and material assets. 3. After evacuating people from the building and moving them outside the danger zone (except employees involved in firefighting), conduct a roll call to confirm who is present. If any employee is missing, immediately determine where they were last seen and organize a search and rescue using the available personnel and equipment. If search operations cannot be conducted because of intense flames or smoke, provide information about the employee’s last known location to the officer in charge of the emergency response unit when the unit arrives at the fire scene. | Responsible official |
| 4 | Meeting the State Fire and Rescue Service response team | 1. Arrange to meet the fire and rescue units, ensure their unobstructed access to the company premises, and help them select the shortest route to the seat of the fire. 2. Assist in briefing the incident commander on the structural and technological features of the premises where the fire occurred and of adjacent buildings and structures. 3. If necessary, arrange for the company’s personnel and resources to be used for measures required to extinguish the fire and prevent it from spreading. | Responsible official |

## Prepared by:

## Occupational Safety Engineer 							     I.P. Surname